|  | # Support Guide |
| --- | --- |

| Subtitle: | How to configure a GPS-Base as an NTRIP server |
| --- | --- |
| Date: | 29/04/2015 |
| Version: | Draft |
| Author: | Simon Nell |
| Document owner: | Support Manager |
| Confidentiality: | None |

List of contents

1. Introduction	1

2. Equipment required	1

3. Method	1

3.1. Setting the baud rate on the GPS-Base	1

3.2. Configuring the GPS-Base	5

3.3. Configuring the laptop	5

3.4. Setting up the NTRIP caster	6

3.5. Connecting to the NTRIP caster	8

4. Revision History	8

# Introduction

This support guide describes to process to configure a GPS-Base as an NTRIP server.

# Equipment required

GPS-Base

GPS-Base power cable

GNSS antenna with cable

Null modem cable

USB/serial adapter

USB A to mini B cable

Laptop

Ethernet cable

NTRIP caster software (available from this download link)

NovAtel config software (available from this download link)

# Method

## Setting the baud rate on the GPS-Base

Connect to the GPS-Base by USB and power it on.

Open the NovAtel config software.

Select the relevant COM port (which will be shown as NovAtel USB Virtual Serial Port), select baud rate 9600 and click Connect

Figure 1: NovAtel config software connected

Select the Configuration tab and click the option 'Type a Command'

Figure 2: NovAtel config software Configuration tab

Enter the following command:

- COM COM2,115200,N,8,1,N,OFF,OFF

Click the 'Send Commands' button

Click the 'Configuration' option, as highlighted in red below.

Figure 3: 'Configuration' option in NovAtel config software

Click 'COM2', as highlighted in red below.

Figure 4: 'COM2' option in NovAtel config software

The next page will show you the baud rate. If the baud rate has not updated, it will show 'Baud Rate (9600,8,N,1, Break Disabled)'. If it has updated correctly, it will show 'Baud Rate (115200,8,N,1, Break Disabled)'

If you wish to save this configuration, then the following command can be sent to the GPS-Base (on the same page as shown in Figure 3):

SAVECONFIG

If the SAVECONFIG command is not sent then the unit will return the baud rate of COM2 to 9600 after the next power cycle.

## Configuring the GPS-Base

- Configure the GPS-Base to use your desired correction format

- Set or average the position of the GPS-Base

- Connect a null modem cable to the COM2 connector of the GPS-Base

- Connect the other end of the null modem cable to your computer (using a USB/serial adapter if necessary)

## Configuring the laptop

- Open Device Manager (searchable from the Start menu)

- Select 'Ports (COM & LPT)', right-click the relevant serial port and click Properties

Figure 5: Device Manager

In the Communications Port window, click the Port Settings tab and set a baud rate of 115200 bps

Figure 6: COM port settings

Connect the computer, via Ethernet, to an externally facing port with a unique IP address. This will allow the NTRIP corrections to be broadcast over the Internet

Open the Network and Sharing Center (searchable from the Start menu)

Click on your Local Area Connection

Click Properties

Double-click Internet Protocol Version 4 (TCP/IPv4)

Select 'Use the following IP address:' and enter the IP address, subnet mask, default gateway and DNS server addresses of your externally facing port.

Click OK to save the IP address configuration

## Setting up the NTRIP caster

- Open the NTRIP caster software

- Select the 'Mount Points' tab

- Enter a name for this caster, and fill in the rest of the details including an approximate lat/lon position

Figure 7: NTRIP caster, Mount Points

Select the 'Users' tab. Add at least 1 user name. You can add as many user names as you like as long as each one is unique. A user name can only be used by one person at a time

Figure 8: NTRIP caster, Users

Select the 'Server' tab. Enter the name of your Mount Point (as shown in Figure 7), select your serial port, 115200 baud rate, and click Connect

Figure 9: NTRIP caster, Server

The NTRIP caster is now outputting data

## Connecting to the NTRIP caster

To connect to an NTRIP caster you will need an NTRIP client. An NTRIP client setup guide is available from support@oxts.com.

- To connect to a caster you will need to following information:

Caster IP address. This will be the IP address the computer was configured with in Section 3.3

Caster port number. This will most likely be port 5000

User name. This is configured in the NTRIP caster software

Password. This is configured in the NTRIP caster software

# Revision History

Table 1

| Revision | Changes |
| --- | --- |
| 150422 | Initial draft |
| 150429 | Final draft |